Buckeye International, Inc.

Material Safety Data Sheet

24 Hour Emergency Telephone Number:    1-800-228-5635    Extension:   076

2700 Wagner Place Maryland Heights, MO 63043 314/291-1900

SECTION I - IDENTIFICATION

'RODUCT NAME

BUCKEYE SPARKLE

| N. F. P. A. 4 = Extreme 3 = High 2 = Moderate 1 = Slight 0 = Insignificant | CJ_P HZA1TK | 2 |
| --- | --- | --- |
|  | £^V IIBI | 0 |
|  | Xf:X REACTTVITY | 0 |

DATE PREPARED

January 1, 1997

CODE

CHEMICAL FAMILY

General Purpose Acid Cleaner Water Based

5260

Corrosive Liquids, n.o.s., (contains Phosphoric Acid), 8, UN 1760, PG III

PROPER D.O.T. SHIPPING NAME

D.O.T. HAZARD CLASSIFICATION

Corrosive Material

SECTION II - INGREDIENTS AND IDENTITY INFORMATION

By WGT

36.3

>62.7

MATERIAL

Phosphoric Acid"

Water

Perfume coloring and additives less than 1%

PEL

1 mg/m

NE

NA

T.L.V.

1 mg/m

NE

NA

C.A.S. NO.

7664-38-2

NE

NA

ITEMS MARKED * ARE SARA TITLE IE SEC 313 REPORTABLES. ALL INGREDIENTS ARE ON TSCA INVENTORY.

SECTION III - PHYSICAL DATA

bOILING POINT °F

212°F

pH (CONC.)

SOLUBILITY IN WATER

% VOLATILE BY WEIGHT

Infinite

63.0

pH (USE DILUTION) 1:128

EVAPORATION RATE (Water=l)

2.3

1.0

SPECIFIC GRAVITY

Liquid

SECTION IV - FIRE AND EXPLOSION DATA

flammable limits

LOWER LIMIT:   NA

APPEARANCE AND ODOR

FLASH POINT (Test Method)

EXTINGUISHING MEDIA

PRODUCT FORM

1.23

Chemical, Clear Purple Solution

Tag Closed Cup: None!

UPPER LIMIT:    NA

NA

SPECIAL FIRE FIGHTING PROCEDURES

Wear acid resistant clothing.

Self-contained breathing apparatus.

UNUSUAL FIRE AND EXPLOSION HAZARDS

Products of combustion.

SECTION V - REACTIVITY DATA

Oxides of carbon and phosphorus.

STABILITY

Stable

CONDITIONS TO AVOID

None known.

INCOMPATIBILITY

Do not mix with chlorine bleach, caustics or alkali. Avoid contact with

ferrous metals and Formica

HAZARDOUS DECOMPOSITION PRODUCTS

None known.

HAZARDOUS POLYMERIZATION						Will not occur.

SECTION VI - HEALTH HAZARD DATA

ROUTE(S) OF ENTRY:	INHALATION? No	SKIN? Yes	INGESTION?   No

HEALTH HAZARDS (Acute and Chronic)

Corrosive. Will cause burns to eyes. Mild skin irritant.

Not toxic by ingestion.

:ARCINOGENICITY:	NTP?   No	IARC MONOGRAPHS?     No	OSHA REGULATED?    No

SIGNS AND SYMPTOMS OF OVEREXPOSURE

For Eyes: Redness or a burning sensation. For Skin: Redness of skin or a warming sensation.

MEDICAL CONDITIONS

GENERALLY AGGRAVATED BY EXPOSURE

None known.

EMERGENCY AND FIRST AID PROCEDURES For Eyes: Flush with cool water for 15 minutes. See physician immediately! For Skin: Wash with soap and

water, if irritation occurs and persists, see physician. For Ingestion: Give two large glasses of water. DO NOT induce vomiting. Call physician. Never give anything by mouth to an unconscious person.

SECTION VII - SPILL OR LEAK PROCEDURES

SPILL RESPONSE
	Dilute with large amount of water. Pick up with mops into a plastic bucket. Wash

area with an alkaline detergent, followed by a clear water rinse. Allow floor to dry before allowing traffic.

WASTE DISPOSAL

METHOD	Dilute with water and adjust pH with caustic to a pH above 6.0, then flush to

sanitary .sewer, or send to sanitary landfill, following local, state and federal laws.

SECTION VIII - SPECIAL PROTECTION INFORMATION

EYE PROTECTION	When eye contact may occur,	VENTILATION

wear safety glasses or chemical splash goggles.	Normal room ventilation.

SKIN	Rubber gloves or other	RESPIRATORY

PROTECTION	impervious gloves.*	PROTECTION	None under normal use.

OTHER

PROTECTION	Footwear that keeps feet dry is suggested.

Follow good personal hygiene practices.

section ix - special precautions

PRECAUTIONS IN	KEEP OUT OF REACH OE CHILDREN!

HANDLING AND STORAGE	Keep container closed when not in use. Store on low shelves.

Store away from caustics and strong alkali. Store at room temperature. Rinse container before discarding.

OTHER	Product is regulated under CERCLA. RQ=5,000 Ibs.

PRECAUTIONS	If more than 10,000 Ibs. is stored in a single day product may require

NA = Noi Applicable	reporting under SARA Title III, Section 311/312 as an immediate health

NE = Not Established       	hazard. Product is regulated under RCRA as a characteristic corrosive.

PREPARED BY:	Mark Gindling, Director of Research

Disclaimer of Liability

As the conditions or methods of use air beyond our control, we do not assume any responsibility and expressly disclaim any liability for any use of the material. Information contained herein is believed to be true and acurate but all statements or suggestions are made without any warranty, express or implied, regarding accuracy oi the information, the hu/.anis connected with the use of the material or the results to be obtained from the use thereof.    Conforms to OS HA 174, Sept 1985